# Otto Sensor — Software Architecture

## 1. System Overview

Otto Sensor is an ESP32-based multi-sensor firmware (Heltec Wireless Stick v2) that reads environmental sensors and publishes telemetry to AWS IoT Core over MQTT with mutual TLS. It supports remote commands, over-the-air (OTA) firmware updates, a Wi-Fi provisioning portal, a local web UI, and LoRa pager output. Current firmware version: 1.0.15.

## 2. Hardware Platform

| Component | Interface | Purpose |
| --- | --- | --- |
| Heltec Wireless Stick v2 (ESP32) | — | MCU, Wi-Fi, OLED display, LoRa radio |
| COZIR CO₂ sensor | UART (Serial2, 9600 baud, pins 32/17) | CO₂ ppm + temperature |
| BME680 (via BSEC2) | I²C (pins 13/21, 100 kHz) | Temp, humidity, pressure, altitude, IAQ/VOC |
| APDS9960 | I²C | Ambient + RGB light |
| ADC (pin 37) | Analog | Battery/input voltage (0–3.3 V) |
| Buzzer (pin 33, LEDC ch 3) | PWM | Audible CO₂ alert (≥800 ppm, 3 s) |
| PWM output (pin 25, LEDC ch 4) | PWM | Remote-controlled output (freq/duty) |
| Power switch (pin 12) | GPIO | Remote on/off control |
| LoRa (915 MHz, SF7, BW125, sync 0xCD) | SPI | Pager notification packets |

## 3. Repository / Module Structure

├── platformio.ini            Build config: board, AWS endpoint flag, embedded certs, lib deps
├── src/
│   ├── main.cpp   (~1,700 ln) Application core: all logic described below
│   ├── OTA.cpp / OTA.h        HTTPS firmware download + flash writer (ESP Update)
│   ├── Watchdog.cpp/.h        Watchdog timer wrapper (reset each loop)
│   ├── Debug.h                DBG_PRINT macros gated by -DDEBUG_LOG
│   └── images.h               OLED bitmap assets
├── lib/
│   ├── COZIR/                 CO₂ sensor UART driver
│   ├── PubSubClient/          MQTT client (vendored)
│   └── ESP32httpUpdate/       Legacy HTTP update helper (vendored)
├── certs/                     Git-ignored: AmazonRootCA1.pem, device cert/key
│                              (embedded into firmware via board_build.embed_txtfiles)
└── tools/update_notifier.py   Desktop OTA notifier (AWS IoT mutual-TLS client)

## 4. Firmware Architecture (main.cpp functional blocks)

### a) Identity & configuration

- get_imei() — derives device ID (IMEI) from MAC; used in MQTT topics and client ID.

- Credentials stored in NVS via Preferences (namespace wifi); fallback to compiled-in defaults.

- AWS IoT endpoint injected at build time (-DAWS_IOT_ENDPOINT); TLS certs embedded as binary symbols.

### b) Network & provisioning

- setup_network() — STA mode with WPA2/PMF-tolerant config (802.11b/g/n forced for router compatibility); on failure starts the captive portal.

- startWiFiSetup(), handleWiFiSetupPage(), handleWiFiSave(), scanNetworksForPortal() — AP-mode portal OttoSensor-<MAC> at 192.168.4.1 with DNS hijack; saves credentials to NVS.

- When connected, a WebServer on port 80 serves the config page and OTA pages; mDNS advertises http://OttoSensors-<id>.local.

### c) Cloud connectivity (MQTT)

- mqtt_connect() — mutual-TLS connect to AWS IoT (port 8883), LWT publishes state:INACTIVE, then publishes state:ACTIVE and subscribes to the command topic. 3 retries, then falls back to the setup portal.

- checkConnections() — non-blocking Wi-Fi/MQTT health check every 10 s (Wi-Fi retry every 15 s).

- Topics (per device):

  - ottosensors/device/<imei>/command (subscribe)

  - ottosensors/device/<imei>/update (telemetry publish)

  - ottosensors/device/<imei>/state (status/LWT publish)

### d) Command handling

- receivedCallback() parses JSON commands: pwm (freq/duty), onoff (power pin), ota (URL+MD5+version), ota_check, ota_install; anything else triggers a telemetry republish.

### e) OTA update system

- runOta() → firmware_ota() (OTA.cpp): streams the image over HTTP(S), verifies MD5, writes to the next OTA slot, reboots on success.

- otaProgress() reports progress via MQTT, OLED progress bar, and keeps the web UI responsive.

- checkForUpdate() polls the GitHub releases API for ottospo/Otto_Sensor every 6 h (or on ota_check); isNewerVersion() compares dotted versions.

- handleUpdatePage/Install/Status — browser-based manual update at /update.

### f) Sensor subsystem

- initSensorCOZIR() / ReadSensorCozir() — CO₂ + temp over UART; fresh-air calibration at boot.

- initSensorBME680() / runBSEC() / newDataCallback() / saveBsecState() — BSEC2 library with periodic state persistence to NVS; produces T/RH/pressure/altitude/IAQ with validity flags.

- initSensorAPDS9960() / ReadSensorAPDS9960() — RGB + clear light.

- setupADC() / ReadVoltageBattery() — battery voltage.

- scanSensorI2CBus() — boot-time I²C diagnostics.

### g) Telemetry & output

- populateMeasurementJson() — builds the JSON payload (temp, humidity, CO₂, VOC/IAQ, RGB/clear, pressure, altitude, voltage, PWM state, power).

- get_data_to_send() — publishes to the update topic.

- sendCo2ToPager() — LoRa packet transmission.

- DisplayMeasurementData() — OLED rendering.

- Buzzer alert logic: CO₂ ≥ 800 ppm → 3 s tone.

## 5. Runtime Model

- setup(): IMEI → Heltec/LoRa/OLED init → Serial2 → ADC → COZIR (+calibration) → I²C bus scan → APDS9960 → BME680/BSEC → PWM/power GPIO → network → MQTT.

- loop() (cooperative, non-blocking, watchdog-fed each iteration):

  - If provisioning: service DNS + web portal only.

  - Otherwise: web server, connection checks, client.loop(), pending OTA, 6-hourly update check, requested publishes.

  - Every pass: BSEC, light sensor, OLED refresh.

  - Every 5 s: CO₂, battery voltage, telemetry publish, LoRa pager, buzzer check.

## 6. Security & Credentials

- AWS IoT mutual TLS (root CA + device cert + private key embedded from git-ignored certs/).

- OTA image integrity via MD5 delivered in the authenticated MQTT command (HTTPS fetch uses setInsecure, relying on MD5 + command-channel auth).

- Wi-Fi credentials and BSEC state persisted in NVS (Preferences).

## 7. Key Dependencies

Heltec ESP32 Dev-Boards 1.1.2 · BSEC2 1.10.2610 · BME68x 1.3.40408 · Adafruit APDS9960 / Unified Sensor · ArduinoJson · PubSubClient (vendored).