Otto Sensor

Firmware Software Architecture

ESP32 firmware • AWS IoT Core • OTA • Wi-Fi provisioning • LoRa

System overview. Otto Sensor is an ESP32-based multi-sensor firmware (Heltec Wireless Stick v2) that reads environmental sensors and publishes telemetry to AWS IoT Core over MQTT with mutual TLS. It supports remote commands, over-the-air (OTA) firmware updates, a Wi-Fi provisioning portal, a local web UI, and LoRa pager output.

| Platform element | Current implementation |
| --- | --- |
| MCU / board | Heltec Wireless Stick v2 (ESP32) |
| Cloud messaging | AWS IoT Core over MQTT with mutual TLS |
| Local services | Wi-Fi provisioning portal, local web UI, OTA pages |
| Auxiliary output | OLED display, buzzer, PWM/power control, LoRa pager |
| Current firmware | 1.0.15 |

Firmware Architecture Reference  |  October 2026

# 1. System Overview

Otto Sensor is an ESP32-based multi-sensor firmware (Heltec Wireless Stick v2) that reads environmental sensors and publishes telemetry to AWS IoT Core over MQTT with mutual TLS. It supports remote commands, over-the-air (OTA) firmware updates, a Wi-Fi provisioning portal, a local web UI, and LoRa pager output. Current firmware version: 1.0.15.

# 2. Hardware Platform

| Component | Interface | Purpose |
| --- | --- | --- |
| Heltec Wireless Stick v2 (ESP32) | — | MCU, Wi-Fi, OLED display, LoRa radio |
| COZIR CO₂ sensor | UART (Serial2, 9600 baud, pins 32/17) | CO₂ ppm + temperature |
| BME680 (via BSEC2) | I²C (pins 13/21, 100 kHz) | Temp, humidity, pressure, altitude, IAQ/VOC |
| APDS9960 | I²C | Ambient + RGB light |
| ADC (pin 37) | Analog | Battery/input voltage (0–3.3 V) |
| Buzzer (pin 33, LEDC ch 3) | PWM | Audible CO₂ alert (≥800 ppm, 3 s) |
| PWM output (pin 25, LEDC ch 4) | PWM | Remote-controlled output (freq/duty) |
| Power switch (pin 12) | GPIO | Remote on/off control |
| LoRa (915 MHz, SF7, BW125, sync 0xCD) | SPI | Pager notification packets |

# 3. Repository / Module Structure

| ├── platformio.ini Build config: board, AWS endpoint flag, embedded certs, lib deps ├── src/ │ ├── main.cpp (~1,700 ln) Application core: all logic described below │ ├── OTA.cpp / OTA.h HTTPS firmware download + flash writer (ESP Update) │ ├── Watchdog.cpp/.h Watchdog timer wrapper (reset each loop) │ ├── Debug.h DBG_PRINT macros gated by -DDEBUG_LOG │ └── images.h OLED bitmap assets ├── lib/ │ ├── COZIR/ CO₂ sensor UART driver │ ├── PubSubClient/ MQTT client (vendored) │ └── ESP32httpUpdate/ Legacy HTTP update helper (vendored) ├── certs/ Git-ignored: AmazonRootCA1.pem, device cert/key │ (embedded via board_build.embed_txtfiles) └── tools/update_notifier.py Desktop OTA notifier (AWS IoT mutual-TLS client) |
| --- |
| Credential handling. The certs/ directory is Git-ignored. AmazonRootCA1.pem and the device certificate/private key are embedded into the firmware via board_build.embed_txtfiles. |  |

# 4. Firmware Architecture (main.cpp functional blocks)

## 4.1 Identity & configuration

get_imei() — derives device ID (IMEI) from MAC; used in MQTT topics and client ID.

Credentials stored in NVS via Preferences (namespace wifi); fallback to compiled-in defaults.

AWS IoT endpoint injected at build time (-DAWS_IOT_ENDPOINT); TLS certs embedded as binary symbols.

## 4.2 Network & provisioning

setup_network() — STA mode with WPA2/PMF-tolerant config (802.11b/g/n forced for router compatibility); on failure starts the captive portal.

startWiFiSetup(), handleWiFiSetupPage(), handleWiFiSave(), scanNetworksForPortal() — AP-mode portal OttoSensor-<MAC> at 192.168.4.1 with DNS hijack; saves credentials to NVS.

When connected, a WebServer on port 80 serves the config page and OTA pages; mDNS advertises http://OttoSensors-<id>.local.

## 4.3 Cloud connectivity (MQTT)

mqtt_connect() — mutual-TLS connect to AWS IoT (port 8883), LWT publishes state:INACTIVE, then publishes state:ACTIVE and subscribes to the command topic. 3 retries, then falls back to the setup portal.

checkConnections() — non-blocking Wi-Fi/MQTT health check every 10 s (Wi-Fi retry every 15 s).

### Per-device MQTT topics

ottosensors/device/<imei>/command (subscribe) ottosensors/device/<imei>/update (telemetry publish) ottosensors/device/<imei>/state (status/LWT publish)

## 4.4 Command handling

receivedCallback() parses JSON commands: pwm (freq/duty), onoff (power pin), ota (URL+MD5+version), ota_check, ota_install; anything else triggers a telemetry republish.

## 4.5 OTA update system

runOta() → firmware_ota() (OTA.cpp): streams the image over HTTP(S), verifies MD5, writes to the next OTA slot, reboots on success.

otaProgress() reports progress via MQTT, OLED progress bar, and keeps the web UI responsive.

checkForUpdate() polls the GitHub releases API for ottospo/Otto_Sensor every 6 h (or on ota_check); isNewerVersion() compares dotted versions.

handleUpdatePage/Install/Status — browser-based manual update at /update.

## 4.6 Sensor subsystem

initSensorCOZIR() / ReadSensorCozir() — CO₂ + temp over UART; fresh-air calibration at boot.

initSensorBME680() / runBSEC() / newDataCallback() / saveBsecState() — BSEC2 library with periodic state persistence to NVS; produces T/RH/pressure/altitude/IAQ with validity flags.

initSensorAPDS9960() / ReadSensorAPDS9960() — RGB + clear light.

setupADC() / ReadVoltageBattery() — battery voltage.

scanSensorI2CBus() — boot-time I²C diagnostics.

## 4.7 Telemetry & output

populateMeasurementJson() — builds the JSON payload (temp, humidity, CO₂, VOC/IAQ, RGB/clear, pressure, altitude, voltage, PWM state, power).

get_data_to_send() — publishes to the update topic.

sendCo2ToPager() — LoRa packet transmission.

DisplayMeasurementData() — OLED rendering.

Buzzer alert logic: CO₂ ≥ 800 ppm → 3 s tone.

# 5. Runtime Model

## setup()

IMEI → Heltec/LoRa/OLED init → Serial2 → ADC → COZIR (+calibration) → I²C bus scan → APDS9960 → BME680/BSEC → PWM/power GPIO → network → MQTT

## loop() — cooperative, non-blocking, watchdog-fed each iteration

If provisioning: service DNS + web portal only.

Otherwise: web server, connection checks, client.loop(), pending OTA, 6-hourly update check, requested publishes.

Every pass: BSEC, light sensor, OLED refresh.

Every 5 s: CO₂, battery voltage, telemetry publish, LoRa pager, buzzer check.

# 6. Security & Credentials

AWS IoT mutual TLS (root CA + device cert + private key embedded from git-ignored certs/).

OTA image integrity via MD5 delivered in the authenticated MQTT command (HTTPS fetch uses setInsecure, relying on MD5 + command-channel auth).

Wi-Fi credentials and BSEC state persisted in NVS (Preferences).

Security implementation note. The current OTA path uses setInsecure for HTTPS fetches and relies on MD5 plus authenticated MQTT command delivery for image integrity/authentication.

# 7. Key Dependencies

| Dependency | Version / source |
| --- | --- |
| Heltec ESP32 Dev-Boards | 1.1.2 |
| BSEC2 | 1.10.2610 |
| BME68x | 1.3.40408 |
| Adafruit APDS9960 / Unified Sensor | — |
| ArduinoJson | — |
| PubSubClient | Vendored |